Circuit Challenge

Names:  _____________________________	_____________________________

_____________________________	_____________________________

| List of Materials: - Alligator Clips - 1.5-3.0 Volt DC Motor - “AA” Battery Holder with 2 AA Batteries | - LED Assembly with Built-In Resistor - Wire - Electrical Tape |
| --- | --- |

FOLLOW THE STEPS:

In order to receive the initials for completion, every team member must show competency.  Be prepared to answer questions.

Initials

______	1.  Set up a SERIES circuit using one (1) battery source, one (1) motor, and one (1) alligator clip.  Remember too much potential difference could fry the motor.

    - Note if the motor is spinning CW or CCW and how fast it is spinning.  Hint: Attach a piece of tape on the end of the motor.

    - Switch the wires attached to the motor.  What do you notice about the direction the motor is spinning?

    - When is the circuit OPEN or CLOSED?

______	2.  Set up a SERIES circuit using one (1) battery source, one (1) LED assembly, and one (1) alligator clip.  The built-in resistor is making sure the LED doesn’t burn out.

    - Note how bright the LED is shining

    - Does the wiring matter?

    - What would happen to the LED if a 9 Volt battery was attached?

______	3.  Add one (1) motor to the SERIES circuit in Step 2.

    - Does the motor appear to be spinning faster or slower when compared to the circuit in Step 1?

    - Does the LED appear to be shinning brighter or dimmer than the circuit in Step 2?

______	4.  Set up a PARALLEL circuit using one (1) battery source, one (1) motor, one (1) LED assembly, and two (2) alligator clips.

    - Disconnect the motor and check if the LED is still lit

    - Disconnect the LED and check if the motor is still spinning

    - Every team member must draw & label ALL electronic components with proper graphical representation along with the potential differences (V), resistances (), & currents (I) for the PARALLEL circuit schematic.  If possible, check your theoretical values with the actual readings of the multi-meter.  Remember, sharing is a virtue.

______	5.  Using parts from another group. connect a COMPLEX circuit that uses both SERIES & PARALLEL.

Once you have completed the challenge, then you can practice soldering at the specified areas.  Don’t forget to wear your goggles.