Name ____________________________

______ /100 points

Physics CWK #14 Circuits

Show your units and work (Given, Find, Known, & Solve).

Equations:

| Series Circuits ΔVtotal=ΔV1+ΔV2+ΔV3+... RTotal = R1 + R2 + R3+… | V=IR P=IΔV P=V2R Parallel Circuits ΔVRTotal=ΔVR1+ΔVR2+ΔVR3+... 1RTotal=1R1+1R2+1R3+... |
| --- | --- |

A

8

20

24

60V

30

Question 1)  In the circuit above, find (a) the combined resistance; (b) the current; and (c) the potential difference across the 20-ohm resistor.

Question 2)  Three resistors are connected in series to a 24-volt battery, and an ammeter in the circuit reads 0.50 amperes.  The first resistor is rated at 22 ohms and the second at 8 ohms.  Find (a) the total resistance; (b) the resistance of the third resistor; (c) the potential difference across the third resistor.

A

60V

Question 3)  The circuit above has the same four resistors as in question 1, but they are connected in parallel.  Find (a) the combined resistance; (b) the current in the battery; (c) the current in the 8-ohm resistor; and (d) the power dissipated in the 24-ohm resistor.

Question 4)  Three resistors are connected in parallel to a 24-volt battery, and the battery current is 3.0 amperes.  The first resistor is rated at 20 ohms and the second at 40 ohms.  Find (a) the total resistance; (b) the resistance of the third resistor; and (c) the current in the third resistor.

Question 5)  Three resistors, of 20 ohms, 40 ohms, and 60 ohms, are connected in parallel to a 24-volt battery of negligible internal resistance.  Find (a) the combined resistance; (b) the current; (c) the power dissipated in the 40-ohm resistance.